Assignment 4 :

GitHub Link : https://github.com/ssbagi/IBM_OpenPOWER_and_SoC_Design/tree/main/DAY2/Assignment%204

README.md File : https://github.com/ssbagi/IBM_OpenPOWER_and_SoC_Design/blob/main/DAY2/Assignment%204/README.md

Question : Design and simulate a 4-bit Up/Down Counter using Verilog HDL.

a) Behavioural modeling.

b) Synchronous reset implementation.

c) Asynchronous reset implementation.

d) Verify up-counting operation.

e) Verify down-counting operation.

- Behavioural modeling :

module up_down_counter_4_bit (

input            clk,

input            reset,

input            up,

output reg [3:0] count

);

always @(posedge clk) begin

if (reset)

count <= 4'b0000;

else if (up)

count <= count + 4'b0001;

else

count <= count - 4'b0001;

end

endmodule

Compilation :

Simulation :

Synchronous reset. In the next clk edge only the count increment is done and stuff.

- Synchronous reset implementation :

Verilog code :

module up_down_counter_4_bit (

input            clk,

input            reset,

input            up,

output reg [3:0] count

);

always @(posedge clk) begin

if (reset)

count <= 4'b0000;

else if (up)

count <= count + 4'b0001;

else

count <= count - 4'b0001;

end

endmodule

Compilation :

Simulation :

- Asynchronous reset implementation :

Verilog code :

module up_down_counter_asynchronous_reset (

input            clk,

input            reset,

input            up,

output reg [3:0] count

);

always @(posedge clk or posedge reset) begin

if (reset)

count <= 4'b0000;

else if (up)

count <= count + 4'b0001;

else

count <= count - 4'b0001;

end

endmodule

Compilation :

Simulation :