Assignment 1 : Combinational Logic Design

- Design and simulate all the six fundamental logic gates OR, NOT, NAND, NOR, XOR, XNOR in Vivado/ Modelsim in dataflow and structural modeling styles.

Dataflow Modelling Code :

/*

Dataflow modelling of the Following gates

1. AND

2. NAND

3. OR

4. NOR

5. NOT

6. XOR

7. XNOR

*/

module and_gate(a, b, y);

input a, b;

output y;

assign y = a & b;

initial $monitor("Time = %0t | AND GATE y = a & b | y = %b", $time, y);

endmodule

module nand_gate(a, b, y);

input a, b;

output y;

assign y = ~(a & b);

initial $monitor("Time = %0t | NAND GATE y = ~(a & b) | y = %b", $time, y);

endmodule

module or_gate(a, b, y);

input a, b;

output y;

assign y = a | b;

initial $monitor("Time = %0t | OR GATE y = (a | b) | y = %b", $time, y);

endmodule

module nor_gate(a, b, y);

input a, b;

output y;

assign y = ~(a | b);

initial $monitor("Time = %0t | NOR GATE y = ~(a | b) | y = %b", $time, y);

endmodule

module not_gate(a, y);

input a;

output y;

assign y = ~a;

initial $monitor("Time = %0t | NOT GATE y = (~a) | y = %b", $time, y);

endmodule

module xor_gate(a, b, y);

input a, b;

output y;

assign y = a ^ b;

initial $monitor("Time = %0t | XOR GATE y = (a ^ b) | y = %b", $time, y);

endmodule

module xnor_gate(a, b, y);

input a, b;

output y;

assign y = ~(a ^ b);

initial $monitor("Time = %0t | XNOR GATE y = ~(a ^ b) | y = %b", $time, y);

endmodule

Structural Modelling Code :

module and_gate(a, b, y);

input a, b;

output y;

and a1(y, a, b);

initial $monitor("Time = %0t | AND GATE y = a & b | y = %b", $time, y);

endmodule

module nand_gate(a, b, y);

input a, b;

output y;

nand n1(y, a, b);

initial $monitor("Time = %0t | NAND GATE y = ~(a & b) | y = %b", $time, y);

endmodule

module or_gate(a, b, y);

input a, b;

output y;

or o1(y, a, b);

initial $monitor("Time = %0t | OR GATE y = (a | b) | y = %b", $time, y);

endmodule

module nor_gate(a, b, y);

input a, b;

output y;

nor n2(y, a, b);

initial $monitor("Time = %0t | NOR GATE y = ~(a | b) | y = %b", $time, y);

endmodule

module not_gate(a, y);

input a;

output y;

not n3(y, a);

initial $monitor("Time = %0t | NOT GATE y = (~a) | y = %b", $time, y);

endmodule

module xor_gate(a, b, y);

input a, b;

output y;

xor x1(y, a, b);

initial $monitor("Time = %0t | XOR GATE y = (a ^ b) | y = %b", $time, y);

endmodule

module xnor_gate(a, b, y);

input a, b;

output y;

xnor x2(y, a, b);

initial $monitor("Time = %0t | XNOR GATE y = ~(a ^ b) | y = %b", $time, y);

endmodule

Testbench :

`ifdef DATAFLOW

`include "logic_gates_dataflow.v"

`else

`include "logic_gates_structure.v"

`endif

module logic_gates_tb;

reg a, b;

wire y_and, y_nand, y_or, y_nor, y_not, y_xor, y_xnor;

and_gate  u_and  (a, b, y_and);

nand_gate u_nand (a, b, y_nand);

or_gate   u_or   (a, b, y_or);

nor_gate  u_nor  (a, b, y_nor);

not_gate  u_not  (a, y_not);

xor_gate  u_xor  (a, b, y_xor);

xnor_gate u_xnor (a, b, y_xnor);

initial begin

$dumpfile("logic_gates_tb.vcd");

$dumpvars(0, logic_gates_tb);

a = 0; b = 0;

#10 a = 0; b = 1;

#10 a = 1; b = 0;

#10 a = 1; b = 1;

#10 $finish;

end

endmodule

Compilation :

iverilog -o sim_structure.vvp logic_gates_tb.v                         # structural (default)

iverilog -DDATAFLOW -o sim_dataflow.vvp logic_gates_tb.v # dataflow

vvp sim_structure.vvp

From the Testbench driven :

Every 10ns there is event change in the inputs a and b. The output changes based on the gates and stuff.

Q2. Design a 2:1 Multiplexer using Verilog HDL in different modeling styles.

- Dataflow Modeling using continuous assignment (assign) statement.

module mux_2_to_1_behavioral (

input wire a,

input wire b,

input wire sel,

output reg y

);

always @(sel, a, b) begin

if (sel)

y = a;

else

y = b;

end

endmodule

- Dataflow Modeling using the conditional (?:) operator.

module mux_2_to_1_behavioral (

input wire a,

input wire b,

input wire sel,

output reg y

);

always @(sel, a, b) begin

if (sel)

y = a;

else

y = b;

end

endmodule

- Behavioural Modeling using always block.

module mux_2_to_1_behavioral_always (

input wire a,

input wire b,

input wire sel,

output reg y

);

always @(a, b, sel) begin

if (sel)

y = a;

else

y = b;

end

endmodule

- Behavioural Modeling using if-else statements.

module mux_2_to_1_behavioral_ifelse (

input wire a,

input wire b,

input wire sel,

output reg y

);

always @(a, b, sel) begin

if (sel)

y = a;

else

y = b;

end

endmodule

- Behavioural Modeling using case statements.

module mux_2_to_1_behavioral_case (

input wire a,

input wire b,

input wire sel,

output reg y

);

always @(*) begin

case (sel)

1'b0 : y = a;

1'b1 : y = b;

endcase

end

endmodule

- Gate-Level Modeling using logic primitives (and, or, not).

module mux_2_to_1_gate_level (

input wire a,

input wire b,

input wire sel,

output wire y

);

wire sel_n, y1, y2;

not(sel_n, sel);

and(y1, sel_n, a);

and(y2, sel, b);

or(y, y1, y2);

endmodule

Compilation of the Mux related Verilog Files :

The VCD dumps of the Mux related ones :