Assignment 6 :

GitHub Link :

README.md Link :

Question : Design and simulate a sequence detector for detecting '1001'.

- Draw the Mealy FSM state diagram.

- Implement the Mealy FSM in Verilog HDL.

Verilog Code :

`timescale 1ns/1ps

module mealy_fsm_detect_1001 (

input  wire clk,

input  wire reset,

input  wire data_in,

output reg  detected

);

localparam [1:0] S0 = 2'b00,

S1 = 2'b01,

S2 = 2'b10,

S3 = 2'b11;

reg [1:0] PS, NS;

always @(posedge clk or posedge reset) begin

if (reset)

PS <= S0;

else

PS <= NS;

end

always @(PS or data_in) begin

NS = S0;

case (PS)

S0: begin

NS = data_in ? S1 : S0;

end

S1: begin

NS = data_in ? S1 : S2;

end

S2: begin

NS = data_in ? S3 : S0;

end

S3: begin

NS = data_in ? S1 : S0;

end

default: NS = S0;

endcase

end

always @(PS or data_in) begin

case (PS)

S0, S1, S2: detected = 1'b0;

S3: detected = data_in;

default: detected = 1'b0;

endcase

end

endmodule

- Draw the Moore FSM state diagram.

Non-Overlapping Case :

Verilog Code :

`timescale 1ns/1ps

module moore_fsm_detect_1001_nonoverlap (

input  wire clk,

input  wire reset,

input  wire data_in,

output reg  detected

);

localparam [2:0] S0 = 3'b000,

S1 = 3'b001,

S2 = 3'b010,

S3 = 3'b011,

S4 = 3'b100;

reg [2:0] PS, NS;

always @(posedge clk or posedge reset) begin

if (reset)

PS <= S0;

else

PS <= NS;

end

always @(PS, data_in) begin

NS = S0;

case (PS)

S0: begin

NS = data_in ? S1 : S0;

end

S1: begin

NS = data_in ? S1 : S2;

end

S2: begin

NS = data_in ? S1 : S3;

end

S3: begin

NS = data_in ? S4 : S0;

end

S4: begin

NS = data_in ? S1 : S0;

end

default: NS = S0;

endcase

end

always @(PS) begin

case (PS)

S0, S1, S2, S3: detected = 1'b0;

S4: detected = 1'b1;

default: detected = 1'b0;

endcase

end

endmodule

Overlapping Case :

Verilog code :

`timescale 1ns/1ps

module moore_fsm_detect_1001_overlap (

input  wire clk,

input  wire reset,

input  wire data_in,

output reg detected

);

localparam [2:0] S0 = 3'b000,

S1 = 3'b001,

S2 = 3'b010,

S3 = 3'b011,

S4 = 3'b100;

reg [2:0] PS, NS;

always @(posedge clk or posedge reset) begin

if (reset)

PS <= S0;

else

PS <= NS;

end

always @(PS, data_in) begin

NS = S0;

case (PS)

S0: begin

NS = data_in ? S1 : S0;

end

S1: begin

NS = data_in ? S1 : S2;

end

S2: begin

NS = data_in ? S1 : S3;

end

S3: begin

NS = data_in ? S4 : S0;

end

S4: begin

NS = data_in ? S1 : S2;

end

default: NS = S0;

endcase

end

always @(PS) begin

case (PS)

S0, S1, S2, S3: detected = 1'b0;

S4: detected = 1'b1;

default: detected = 1'b0;

endcase

end

endmodule