Assignment 5 :

- Github Link :

- README.md Link :

Question 1 : Mux 2 to 1

Verilog code :

module mux_2_to_1_behavioral (

input wire a,

input wire b,

input wire sel,

output reg y

);

always @(sel, a, b) begin

if (sel)

y = a;

else

y = b;

end

endmodule

Testbench code :

`include "mux_2_to_1_behavioral.v"

module mux_2_to_1_behavioral_tb;

reg a, b, sel;

wire y;

mux_2_to_1_behavioral uut (.a(a), .b(b), .sel(sel), .y(y));

initial begin

$dumpfile("mux_2_to_1_behavioral_tb.vcd");

$dumpvars(0, mux_2_to_1_behavioral_tb);

$monitor("Time = %0t | a=%b b=%b sel=%b | y=%b", $time, a, b, sel, y);

#0   a = 0; b = 0; sel = 0;

#10 a = 0; b = 1; sel = 0;

#10 a = 1; b = 0; sel = 0;

#10 a = 1; b = 1; sel = 0;

#10 a = 0; b = 0; sel = 1;

#10 a = 0; b = 1; sel = 1;

#10 a = 1; b = 0; sel = 1;

#10 a = 1; b = 1; sel = 1;

#10 $finish;

end

endmodule

Compilation and Simulation Snapshot :

From the screenshot we observe for every 10 nanoseconds there is change in the output variable y. The selection when sel = 0 then we select the variable b and when sel = 1 then we select the variable a.

Waveform Window Snapshot : The same above what is observed in the log file even captured in the waveform window also.

Question 2 :

Design a self-checking testbench for a 4-bit Magnitude Comparator and simulate using Vivado.

Verilog Code :

module comparator_4bit (

input  [3:0] a,

input  [3:0] b,

output reg  a_greater,

output reg  b_greater,

output reg  equal

);

always @(*) begin

a_greater = 1'b0;

b_greater = 1'b0;

equal = 1'b0;

if (a[3] != b[3]) begin

a_greater = a[3];

b_greater = b[3];

end else if (a[2] != b[2]) begin

a_greater = a[2];

b_greater = b[2];

end else if (a[1] != b[1]) begin

a_greater = a[1];

b_greater = b[1];

end else if (a[0] != b[0]) begin

a_greater = a[0];

b_greater = b[0];

end else begin

equal = 1'b1;

end

end

endmodule

Testbench Code :

`timescale 1ns/1ps

module comparator_4bit_tb;

reg  [3:0] a;

reg  [3:0] b;

wire       a_greater;

wire       b_greater;

wire       equal;

integer input_value_a;

integer input_value_b;

integer random_value_a;

integer random_value_b;

integer random_test_index;

integer random_seed;

integer test_count;

integer failure_count;

comparator_4bit dut (

.a(a),

.b(b),

.a_greater(a_greater),

.b_greater(b_greater),

.equal(equal)

);

task automatic check_case;

input [3:0] test_a;

input [3:0] test_b;

begin

a = test_a;

b = test_b;

#1;

test_count = test_count + 1;

if (test_a > test_b) begin

$display("Test %0d: A=%0d, B=%0d -> A > B | result: a_greater=%b b_greater=%b equal=%b",

test_count, test_a, test_b, a_greater, b_greater, equal);

if ({a_greater, b_greater, equal} !== 3'b100) begin

$display("FAIL: expected A > B, got %b", {a_greater, b_greater, equal});

failure_count = failure_count + 1;

end

end else if (test_a < test_b) begin

$display("Test %0d: A=%0d, B=%0d -> A < B | result: a_greater=%b b_greater=%b equal=%b",

test_count, test_a, test_b, a_greater, b_greater, equal);

if ({a_greater, b_greater, equal} !== 3'b010) begin

$display("FAIL: expected A < B, got %b", {a_greater, b_greater, equal});

failure_count = failure_count + 1;

end

end else begin

$display("Test %0d: A=%0d, B=%0d -> A == B | result: a_greater=%b b_greater=%b equal=%b",

test_count, test_a, test_b, a_greater, b_greater, equal);

if ({a_greater, b_greater, equal} !== 3'b001) begin

$display("FAIL: expected A == B, got %b", {a_greater, b_greater, equal});

failure_count = failure_count + 1;

end

end

end

endtask

initial begin

$dumpfile("4bit_comparator_tb.vcd");

$dumpvars(0, comparator_4bit_tb);

test_count = 0;

failure_count = 0;

random_seed = 32'h1ACE_B00C;

$display("Starting 20 deterministic random comparator tests...");

for (random_test_index = 0; random_test_index < 20; random_test_index = random_test_index + 1) begin

random_value_a = $random(random_seed) & 4'hf;

random_value_b = $random(random_seed) & 4'hf;

check_case(random_value_a, random_value_b);

end

$display("Starting 256-case exhaustive comparator test...");

for (input_value_a = 0; input_value_a < 16; input_value_a = input_value_a + 1) begin

for (input_value_b = 0; input_value_b < 16; input_value_b = input_value_b + 1) begin

check_case(input_value_a, input_value_b);

end

end

if (failure_count == 0) begin

$display("PASS: %0d random and exhaustive comparator tests completed successfully.", test_count);

end else begin

$display("FAIL: %0d of %0d comparator tests failed.", failure_count, test_count);

$fatal(1);

end

$finish;

end

endmodule

Compilation and Simulation :

Simulation Waveform :

Question 3 :

Write a complete testbench for a D Flip-Flop with synchronous reset and simulate in Vivado.

Verilog code :

Testbench code :

Compilation and Simulation Output :

Simulation Waveform :

First two clk edges rst is applied then we see the D Flip Flop is Synchronous reset.

At clk = 25ns to 100ns we observe that d = 1 on every posedge of the clock and stuff hence we see q =1 and qbar = 0. At clk = 105ns we observe that d = 0 on every posedge of the clock and stuff hence we see q = 0 and qbar = 1.

Question 4 :

Write and simulate a testbench for the 4-bit Up/Down Counter (Assignment 04) in Vivado.

Verilog Code :

module up_down_counter_asynchronous_reset (

input            clk,

input            reset,

input            up,

output reg [3:0] count

);

always @(posedge clk or posedge reset) begin

if (reset)

count <= 4'b0000;

else if (up)

count <= count + 4'b0001;

else

count <= count - 4'b0001;

end

endmodule

Testbench Code :

/* Testbench for the asynchronous-reset up/down counter. */

`timescale 1ns/1ps

module up_down_counter_asynchronous_reset_tb;

reg clk;

reg reset;

reg up;

wire [3:0] count;

integer test_count;

integer failure_count;

integer value;

reg [87:0] mode;

up_down_counter_asynchronous_reset dut (

.clk(clk),

.reset(reset),

.up(up),

.count(count)

);

always #5 clk = ~clk;

task check_count;

input [3:0] expected_count;

begin

#1;

test_count = test_count + 1;

if (reset)

mode = "ASYNC RESET";

else if (up)

mode = "UP COUNT";

else

mode = "DOWN COUNT";

if (count !== expected_count) begin

$display("FAIL: Test %0d | %s | Expected=%b | Got=%b",

test_count, mode, expected_count, count);

failure_count = failure_count + 1;

end else begin

$display("PASS: Test %0d | %s | Count=%b",

test_count, mode, count);

end

end

endtask

initial begin

$dumpfile("4_bit_up_down_asynchronous_reset_tb.vcd");

$dumpvars(0, up_down_counter_asynchronous_reset_tb);

clk = 1'b0;

reset = 1'b0;

up = 1'b1;

test_count = 0;

failure_count = 0;

$display("====================================================");

$display("Asynchronous Reset 4-bit Up/Down Counter Test     ");

$display("====================================================");

#2 reset = 1'b1;

check_count(4'd0);

up = 1'b0;

#2 reset = 1'b1;

check_count(4'd0);

reset = 1'b0;

up = 1'b1;

$display("\nTesting all 16 UP COUNT states...");

for (value = 1; value <= 16; value = value + 1) begin

@(posedge clk);

check_count(value[3:0]);

end

up = 1'b0;

$display("\nTesting all 16 DOWN COUNT states...");

for (value = 15; value >= 0; value = value - 1) begin

@(posedge clk);

check_count(value[3:0]);

end

if (failure_count == 0)

$display("\nPASS: %0d/%0d asynchronous-reset tests passed.",

test_count, test_count);

else

$fatal(1, "FAIL: %0d of %0d tests failed.",

failure_count, test_count);

$finish;

end

endmodule

Compilation and Simulation Output :

Simulation Waveform :

The Up Count when up = 1 and Down Count when up = 0.

Asynchronous Reset :

If we observe as at the Time = 2ns. We make the reset = 1 irrespective of the clock edge the changes are taken into effect hence we see count value transition from the Logic X to Logic 0.